Title: Simulate a Full Adder in VHDL.

Problem Description: Develop a program in Verilog to perform full adder operation.

Full adder

module Fulladder12(sum,cout,a,b,c);

input a,b,c;

output sum,cout;

assign sum=a^b^c;

assign cout=(a&b)|(b&c)|(c&a);

endmodule;

Title: Simulate a 8:1 MUX in VHDL

Problem Description: Develop a program in VHDL to perform 8:1 multiplexer operation.

module m81(out, D0, D1, D2, D3, D4, D5, D6, D7, S0, S1, S2);

input wire D0, D1, D2, D3, D4, D5, D6, D7, S0, S1, S2;

output reg out;

always@(*)

begin

case({S0,S1,S2})

3'b000: out=D0;

3'b001: out=D1;

3'b010: out=D2;

3'b011: out=D3;

3'b100: out=D4;

3'b101: out=D5;

3'b110: out=D6;

3'b111: out=D7;

default: out=1'b0;

endcase

end

endmodule

Title: Simulate a Flip-Flops in Verilog.

Problem Description: Develop a program in verilog to check the working of flip-flops (JK/D/T)

module jk_ff ( input j, input k, input clk, output reg q);

initial begin

q=0;

end

always @ (posedge clk)

begin

case ({j,k})

2'b00 :  q <= q;

2'b01 :  q <= 0;

2'b10 :  q <= 1;

2'b11 :  q <= ~q;

endcase

end

endmodule

Counter

Title: Simulate a 4-bit binary counter VHDL program.

Problem Description: Develop a program in VHDL to check 4-bit binary counter.

module counter (input clk,      // Declare input port for the clock to allow counter to count up

input rstn,              // Declare input port for the reset to allow the counter to be reset to 0 when required

output reg[3:0] out);    // Declare 4-bit output port to get the counter values

// This always block will be triggered at the rising edge of clk (0->1)

// Once inside this block, it checks if the reset is 0, then change out to zero

// If reset is 1, then the design should be allowed to count up, so increment the counter

always @ (posedge clk) begin

if (! rstn)

out <= 0;

else

out <= out + 1;

end

endmodule

Ripple carry adder

Title: Simulate a 4-bit binary counter VHDL program.

Problem Description: Develop a program in VHDL to check 4-bit binary counter.

module rippe_adder(X, Y, S, Co);

input [3:0] X, Y;// Two 4-bit inputs

output [3:0] S;

output Co;

wire w1, w2, w3;

// instantiating 4 1-bit full adders in Verilog

fulladder u1(X[0], Y[0], 1'b0, S[0], w1);

fulladder u2(X[1], Y[1], w1, S[1], w2);

fulladder u3(X[2], Y[2], w2, S[2], w3);

fulladder u4(X[3], Y[3], w3, S[3], Co);

endmodule

module Fulladder12(a,b,c,sum,cout,);

input a,b,c;

output sum,cout;

assign sum=a^b^c;

assign cout=(a&b)|(b&c)|(c&a);

endmodule;

4 bit ALU

Title: Design of a 4-bit ALU.

Problem Description: Develop a program in VHDL to perform 4-bit ALU.

module alu(

input [7:0] A,B,  // ALU 8-bit Inputs

input [3:0] ALU_Sel,// ALU Selection

output [7:0] ALU_Out, // ALU 8-bit Output

output CarryOut // Carry Out Flag

);

reg [7:0] ALU_Result;

wire [8:0] tmp;

assign ALU_Out = ALU_Result; // ALU out

assign tmp = {1'b0,A} + {1'b0,B};

assign CarryOut = tmp[8]; // Carryout flag

always @(*)

begin

case(ALU_Sel)

4'b0000: // Addition

ALU_Result = A + B ;

4'b0001: // Subtraction

ALU_Result = A - B ;

4'b0010: // Multiplication

ALU_Result = A * B;

4'b0011: // Division

ALU_Result = A/B;

4'b0100: // Logical shift left

ALU_Result = A<<1;

4'b0101: // Logical shift right

ALU_Result = A>>1;

4'b0110: // Rotate left

ALU_Result = {A[6:0],A[7]};

4'b0111: // Rotate right

ALU_Result = {A[0],A[7:1]};

4'b1000: //  Logical and

ALU_Result = A & B;

4'b1001: //  Logical or

ALU_Result = A | B;

4'b1010: //  Logical xor

ALU_Result = A ^ B;

4'b1011: //  Logical nor

ALU_Result = ~(A | B);

4'b1100: // Logical nand

ALU_Result = ~(A & B);

4'b1101: // Logical xnor

ALU_Result = ~(A ^ B);

4'b1110: // Greater comparison

ALU_Result = (A>B)?8'd1:8'd0 ;

4'b1111: // Equal comparison

ALU_Result = (A==B)?8'd1:8'd0 ;

default: ALU_Result = A + B ;

endcase

end

endmodule

| Alu_SET | Operation | ALU_Result |
| --- | --- | --- |
| 4'b0000: // Addition | A+B |  |
| 4'b0001: // Subtraction | A-B |  |
| 4'b0010: // Multiplication | A * B |  |
| 4'b0011: // Division | A/B; |  |
| 4'b0100: // Logical shift left | A<<1; |  |
| 4'b0101: // Logical shift right | A>>1 |  |
| 4'b0110: // Rotate left | {A[6:0],A[7]} |  |
| 4'b0111: // Rotate right | {A[0],A[7:1]}; |  |
| 4'b1000: // Logical and | A & B |  |
| 4'b1001: // Logical or | A ¦ B |  |
| 4'b1010: // Logical xor | A^B |  |
| 4'b1011: // Logical nor | ~(A ¦ B) |  |
| 4'b1100: // Logical nand | ~(A & B) |  |
| 4'b1101: // Logical xnor | ~(A ^ B) |  |
| 4'b1110: // Greater comparison | (A>B) |  |
| 4'b1111: // Equal comparison | (A==B) |  |