- Ripple carry adder

Title: Simulate a 4-bit binary counter Verilog program.

Problem Description: Develop a program in Verilog to check 4-bit binary counter.

module rippe_adder(X, Y, S, Co);

input [3:0] X, Y;// Two 4-bit inputs

output [3:0] S;

output Co;

wire w1, w2, w3;

// instantiating 4 1-bit full adders in Verilog

fulladder u1(X[0], Y[0], 1'b0, S[0], w1);

fulladder u2(X[1], Y[1], w1, S[1], w2);

fulladder u3(X[2], Y[2], w2, S[2], w3);

fulladder u4(X[3], Y[3], w3, S[3], Co);

endmodule

module Fulladder12(a,b,c,sum,cout);

input a,b,c;

output sum,cout;

assign sum=a^b^c;

assign cout=(a&b)|(b&c)|(c&a);

endmodule;

- 4 bit ALU

Title: Design of a 4-bit ALU.

Problem Description: Develop a program in Verilog to perform 4-bit ALU.

module alu(

input [7:0] A,B,  // ALU 8-bit Inputs

input [3:0] ALU_Sel,// ALU Selection

output [7:0] ALU_Out, // ALU 8-bit Output

output CarryOut // Carry Out Flag

);

reg [7:0] ALU_Result;

wire [8:0] tmp;

assign ALU_Out = ALU_Result; // ALU out

assign tmp = {1'b0,A} + {1'b0,B};

assign CarryOut = tmp[8]; // Carryout flag

always @(*)

begin

case(ALU_Sel)

4'b0000: // Addition

ALU_Result = A + B ;

4'b0001: // Subtraction

ALU_Result = A - B ;

4'b0010: // Multiplication

ALU_Result = A * B;

4'b0011: // Division

ALU_Result = A/B;

4'b0100: // Logical shift left

ALU_Result = A<<1;

4'b0101: // Logical shift right

ALU_Result = A>>1;

4'b0110: // Rotate left

ALU_Result = {A[6:0],A[7]};

4'b0111: // Rotate right

ALU_Result = {A[0],A[7:1]};

4'b1000: //  Logical and

ALU_Result = A & B;

4'b1001: //  Logical or

ALU_Result = A | B;

4'b1010: //  Logical xor

ALU_Result = A ^ B;

4'b1011: //  Logical nor

ALU_Result = ~(A | B);

4'b1100: // Logical nand

ALU_Result = ~(A & B);

4'b1101: // Logical xnor

ALU_Result = ~(A ^ B);

4'b1110: // Greater comparison

ALU_Result = (A>B)?8'd1:8'd0 ;

4'b1111: // Equal comparison

ALU_Result = (A==B)?8'd1:8'd0 ;

default: ALU_Result = A + B ;

endcase

end

endmodule

| Alu_SET | Operation | ALU_Result |
| --- | --- | --- |
| 4'b0000: // Addition | A+B |  |
| 4'b0001: // Subtraction | A-B |  |
| 4'b0010: // Multiplication | A * B |  |
| 4'b0011: // Division | A/B; |  |
| 4'b0100: // Logical shift left | A<<1; |  |
| 4'b0101: // Logical shift right | A>>1 |  |
| 4'b0110: // Rotate left | {A[6:0],A[7]} |  |
| 4'b0111: // Rotate right | {A[0],A[7:1]}; |  |
| 4'b1000: // Logical and | A & B |  |
| 4'b1001: // Logical or | A ¦ B |  |
| 4'b1010: // Logical xor | A^B |  |
| 4'b1011: // Logical nor | ~(A ¦ B) |  |
| 4'b1100: // Logical nand | ~(A & B) |  |
| 4'b1101: // Logical xnor | ~(A ^ B) |  |
| 4'b1110: // Greater comparison | (A>B) |  |
| 4'b1111: // Equal comparison | (A==B) |  |